EXHIBIT VII - APPENDIX 1

REQUIREMENTS FOR MATERIALS TRACEABILITY

FPSO PETROBRAS 91 (P-91)

************Revision Control*************

Rev 0: Bid original version

Objective

This appendix establishes the supplementary requirements to ISO 9001 for traceability of materials during manufacturing, construction and assembly to be fulfilled by Seller and its supply chain.

The Buyer reserves the right to check the application and maintenance of the requirements herein defined, at any time, including those related to suppliers and subcontractors. This may include diligences, inspections, audits and application of checklists.

Reference Documents

- ISO 9001:2015 – Quality Management System – Requirements

Definitions and Abbreviatures

- ISO International Organization for Standardization

- NDT Non-Destructive Testing

- FRP Fiber Reinforced Polymers

Type of Traceability

Full Traceability (“FT”) - Material is uniquely identified and its history (quality certificate) tracked from manufacture (mill) through stockist (where applicable) and throughout fabrication, construction and assembly stating final position of the component with specific location defined on a material placement record.

- NOTE: The material placement record (correlation between material quality certificates and individual component position) shall be included in data book.

Limited Traceability (“LT”) - Material is tracked throughout fabrication, construction and assembly, with traceability to a material quality certificate.

Recognized Origin (“RO”) - Material purchased from a qualified supplier with traceability that assure compliance of the component with design.

V. Requirements per Materials

Material Certificates are required for all materials being received. Receiving Inspection shall be performed including the verification of the manufacturing certificate, according to the applicable standard.

Spare parts shall comply with the same requirements of related main parts.

Seller and vendor and subcontractors shall comply with the following rules for the traceability of materials presented in Tables 1 to 9.

Complementary Notes shall be considered according to specific materials.

Traceability requirements for items not listed shall be agreed between Buyer and Seller.

Table 1 - General Requirements.

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Bolts, Nuts, Washers (Note 1) | - | - | X |
| Gaskets (Note 2) and O-rings | - | - | X |
| Insulating and refractory consumables | - | - | X |
| Painting consumables (paint, varnish and solvents) | - | X | - |
| NDE consumables | - | - | X |
| Welding Filler Metal and Flux | X (Note 3) | - | X |
| Welding Gas and other Consumables |  | - | X |
| Valves (manual or actuated) | X | - | - |
| Safety Valves | X | - | - |
| Fire Safe Materials | X | - | - |
| Composite Materials | - | - | X |

Table 2 - Piping Requirements.

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Pressure retaining parts (pipes, forgings, castings) | X (Note 4) | - | - |
| Expansion Joints | X | - | - |
| Special Supports | X | - | - |
| Spring Hangers | X | - | - |
| Hoses | - | - | X |
| Quick release couplings | - | - | X |
| Welded Joints (Note 5) | X (Note 4) |  |  |

Table 3 – Static Equipment (pressure vessels, heat exchangers, tanks, filters, flare and so on).

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Pressure retaining parts | X |  |  |
| Internals | X |  |  |
| Welded Joints (Note 5) | X (Note 4) | X |  |

Table 4 – Rotating Equipment, Cranes, Mechanical Parts and Pull in System.

| EQUIPMENTS AND COMPONENTS |  |  |  |
| --- | --- | --- | --- |
| Rotative Compressors, Gear Boxes, VSFD and Rotative Pumps. | FT | LT | RO |
| Casing, Nozzles and Inner Case Parts | X |  |  |
| Rotating Elements and Wear Rings | X |  |  |
| Welded Joints and Repairs (Note 5) | X |  |  |
| Bearings |  | X |  |
| Bearing Housings |  | X |  |
| Cranes | FT | LT | RO |
| Primary Structure | X |  |  |
| Lifting Equipment-Hook. Hook Block and Ropes | X |  |  |
| Critical Components (Note 6) |  | X |  |
| Hoses (Hydraulic and Pneumatic) |  | X |  |
| Critical Fasteners |  | X |  |
| Welded Joints for Primary Structures (Note 5) | X |  |  |
| Mechanical Parts (Note 7) | FT | LT | RO |
| Steel Wires |  | X |  |
| Fuels and Lubricants |  |  | X |
| Couplings |  | X |  |
| Seal (Gland and Sleeve) | X |  |  |
| Seal (other parts) |  |  | X |
| Pull-in System | FT | LT | RO |
| Anchor | X |  |  |
| Mooring Chain and Accessories |  |  | X |
| Steel Wire Ropes |  |  | X |
| Steel Wire Rope End Attachment |  |  | X |
| Fiber Rope Segments and Termination Hardware |  |  | X |
| Winch Drum and Drum Flanges | X |  |  |
| Windlass/Winch Framework | X |  |  |
| Shafts for Cable Lifter and/or Drum | X |  |  |
| Couplings |  | X |  |
| Gear Shafts and Gear Wheels | X |  |  |
| Brake Components (Pawl Wheel/Stopper) | X |  |  |
| Chain Stopper | X |  |  |
| Mooring Line Buoyancy Element (MLBE) | X |  |  |
| Fairlead (Cable Lifter/Sheaves, Shafts, Housing and Support) |  | X |  |
| Towing Equipment (Shackles, Flounder Plate, Steel Wire Rope and Chain) |  | X |  |
| Welded Joints (Note 5) | X |  |  |

Table 5 – Electricity / communications

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| General use Materials |  |  | X |
| Electrical cables |  | X |  |
| Trays and supports |  | X (Note 8) | X |
| MCT |  |  | X |
| IP or Ex Components (lightening, junction box, connectors, cable gland and so on) |  | X |  |
| Motors and Generators Materials |  | X |  |
| Transformers Materials |  | X |  |
| Panels Materials |  |  | X |
| UPS Materials |  | X |  |
| Batteries Materials |  | X |  |

Table 6 – Instrumentation.

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Cables and multiple cables |  | X |  |
| Valves (hydraulic/pneumatic systems) |  | X |  |
| Connectors and end fittings |  |  | X |
| Junctions Box (with IP or Ex) |  | X |  |
| Trays and supports |  | X (Note 8) | X |
| Small Bore Tubing System (Tubes, Fittings, tubing valves and air manifolds) |  | X |  |
| Solenoids, contactors |  |  | X |
| Instruments Materials |  | X |  |

Table 7 – Safety.

| MATERIALS / COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Fire extinguishers |  |  | X |
| Hydraulic systems |  |  | X |
| Electronic systems |  |  | X |
| Mechanical Equipment |  |  | X |
| Life buoys and jackets |  |  | X |
| Flash lights |  |  | X |
| Light and smokes signaling devices |  |  | X |
| Metallic and nonmetallic fabric |  |  | X |
| Hoses |  |  | X |
| Masks, fiber glass boxes |  |  | X |

Table 8 – Mooring System.

| MATERIALS | FT | LT | RO |
| --- | --- | --- | --- |
| Hook up/ installation chains | X |  |  |
| Fairleads | X |  |  |
| Chain-stoppers | X |  |  |
| Hook-up/installation wire ropes | X |  |  |

Table 9 – Structure.

| STRUCTURAL COMPONENTS | FT | LT | RO |
| --- | --- | --- | --- |
| Hull and Modules Plates | X |  |  |
| Hull and Modules Beams | X |  |  |
| Hull Longitudinal Profiles | X |  |  |
| Equipment Skids |  | X |  |
| Walkways, stairs, handrails, piping supports, attachments |  | X (Note 8) | X |
| Welded Joints (Note 5) | X (Note 9) | X |  |

Complementary Notes:

Note 1: All bolts and nuts shall be properly hard stamped (by the manufacturer) according to the applicable standard. This is intended to properly identify and avoid misuse of regular bolts and special purpose bolts (e.g. low temperature, H2S service, high yield strength).

Note 2: All gaskets shall be properly marked and color coded as required by the applicable standards.

Note 3: FT for welding filler metal and flux is required as per welded joints for concerned equipment, piping and structure requirements.

Note 4: FT is required on the following cases:

a) Carbon steel piping for minimum design temperature below -29ºC.

b) Alloy steel (low, medium, high alloy, including stainless steel grades), and all non-ferritic materials (Cu-Ni, Inconel, Titanium, Hastelloy, and so on).

c) Pressure rating #900 and above.

d) H2S and amine service.

Note 5: When FT is required for welded joints it shall be traceable to the welder, welding procedure specification (WPS), filler metal certificate, and NDT reports applicable.

Note 6: A critical component is any component of the crane assembly devoid of redundancy and auxiliary restraining devices, whose failure results in an uncontrolled movement of the crane or load.

Note 7: Materials are considered purchased individually, independently of equipment purchase. For materials that are part of the equipment, the supplier shall guarantee the traceability of every material.

Note 8: LT is required for FRP materials.

Note 9: FT is required for welded joints which volumetric NDT is required.