EXHIBIT XV

DIRECTIVES FOR FPSO CLASSIFICATION

FPSO PETROBRAS 91 (P-91)

************Revision Control*************

Rev 0: Bid original version

SUMMARY

1.	GENERAL	3

2.	REFERENCE DOCUMENTS	4

3.	CLASS NOTATION	4

4.	SPECIFIC REQUIREMENTS	5

5.	CERTIFICATES, TERMS AND DECLARATIONS	9

# GENERAL

This Directive defines the conditions to be followed by SELLER, and Vendors regarding the Classification Society services for the FPSO Class assignment. SELLER will provide the Certification for the entire UNIT, including engineering design, acquisition, hull, construction and assembly of modules and topside, lifting and integration of modules and hull, commissioning, testing, pre-operation and operation, until the transfer of the UNIT to BUYER, in order to obtain and maintain the appropriate classification of the UNIT.

The Classification Society shall be one of the following: Bureau Veritas (BV), Det Norske Veritas (DNV), Lloyd’s Register of Shipping (LRS) or American Bureau of Shipping (ABS). BUYER may substitute the Classification Society prior to start operation and SELLER shall support it with all required documentation. The Classification Society shall be unique for all parts and systems of the UNIT.

The FPSO shall be designed and built by SELLER in accordance with all applicable Rules and Regulations of Classification Society, International Conventions and Flag State Administration requirements, hereinafter referred to as Rules and Regulations.

The FPSO will be defined and registered by BUYER, at a convenient moment, no later than 18 (eighteen) months prior to FPSO´s delivery date, under one of the following Flag States: Marshall Islands or Liberia. SELLER or/and assigned Class Society must provide the necessary Statements, documents or Certifications in order to support BUYER to conclude the Registry of the FPSO. The analysis and surveys will be carried out to the limits that the Classification Society is authorized by the Flag State Administration. All corresponding fee and charges related to register on the flag, shall be borne by BUYER.

The riser systems connected to the FPSO shall not be classified.

Surveys and issuance of all necessary Statutory Certificates is within Classification Society scope of work.

The Contract between SELLER and the Classification Society shall be sent to BUYER, prior to its signature, according to the following procedure:

The Contract minutes, redacted contract amount is acceptable, containing all contractual clauses, terms and conditions, plans and items shall be submitted by SELLER for BUYER’s analysis up to 40 (forty) calendar days after Agreement Effective Date.

SELLER shall sign its Contract with the Classification Society up to 50 (fifty) calendar days after the AGREEMENT Effective Date and issue a copy to BUYER.

# REFERENCE DOCUMENTS

IMO (International Maritime Organization) MODU (Mobile Offshore Drilling Units) Code and other pertinent regulations and conventions, such as:

Marpol (International Convention for the Prevention of Pollution from Ships) and Its applicable Annexes: (Consolidated Edition 2020 along with MEPC.311(73), 2018 Guidelines for the Application of Marpol Annex I Requirements to Floating Production, Storage and Offloading Facilities (FPSOS) and Floating Storage Units (FSUS)); MARPOL 73/78 (International Convention for the Prevention of Pollution from Ships) - Consolidated Edition 2020 along with MEPC.311(73), 2018 Guidelines for the Application of Marpol Annex I Requirements to Floating Production, Storage and Offloading Facilities (FPSOS) and Floating Storage Units (FSUS).

International Convention on Load Lines (CLL) (up to amendments in Resolution A.1083(30) and Circular LL.12/Circ.1.)

SOLAS (International Convention for the Safety of Life at Sea) (as amended in the Consolidated Edition 2020)

MODU, 2009 (Code For The Construction And Equipment of Mobile Offshore Drilling Units) up to the amendments in MSC.407(96) and MSC.435(98)

COLREG (Convention on the International Regulations for Preventing Collisions at Sea) – up to 2016 Amendments Res A.1085(28)

ILO (International Labour Organization) Conventions 92 (1949) and 133 (1949), 152 (1979).

Classification Society FPSO Rules:

ABS: Rules for Building and Classing - Floating Production Installations, or

BV: Rules for the Classification of Offshore Units, or

DNV: Rules for Classification - Offshore Units - Floating Production, Storage and Loading Units, or

LRS: Rules for the Classification of Offshore Units

Flag State Administration Rules and Regulations (Marshall Islands 2024 or Liberia)

Brazilian Navy Standards issued by Ports and Coasts Directorate (Mainly NORMAM 201 2024, NORMAM 203 2024 and NORMAM 223 2024).

# CLASS NOTATION

Final Notation of Class for the integrated FPSOs shall be according to one of the following:

American Bureau of Shipping (ABS):

✠A1, Floating Production, Storage and Offloading System (Ship-Type), XXXX Basin XXXX Field, FL(XX), uwild, SUSTAIN 1, IHM

Bureau Veritas – BV:

I ✠ HULL ✠MACH Offshore Service Barge / Oil Storage / Production XXXX Basin – XXXX Field / PERMANENT POSA / PROC / FAT(XX) /inwatersurvey/ sustainable ship-1

DNV:

✠ OI Ship Shaped Hydrocarbon Production and Storage unit FMS(30) Field(XXXX)  BIS POSMOOR PROD Recyclable

Lloyd’s Register:

✠OI100AT Floating Production Storage & Offloading Unit, ShipRight (SDA, FDA(XX years TBA), CM), OWIS, mooring description, PPF, ECO for service at XXXX Basin XXXX Field

NOTE: Regarding the additional class Notations above, it is necessary to indicate the requested fatigue life (XX) according to project requirements.

Not applicable

# SPECIFIC REQUIREMENTS

The Contract with the Classification Society shall include clear and specific terms, clauses and conditions assuring that the specific requirements, activities and services described herein (items 4.1 to 4.5) are fulfilled.

Planning and Control for FPSO Classification

The Classification Society shall nominate a focal point to be responsible for all relevant communications with BUYER.

The Classification Society shall nominate a technical group, identifying the name responsible for each engineering discipline and classification activity during all phases: Detailed Engineering, Construction & Assembly, Transport and Commissioning.

The Classification Society shall nominate a dedicated Surveyor at each construction site.

The Classification Society shall nominate a dedicated Engineering Project Leader.

Not Applicable

The Classification Society shall nominate a Main Office Senior Representative for arbitration, clarification and technical advice, to be called in case of major disagreements between BUYER and SELLER regarding compliance with Rules and Regulations that can be raised at any time during Engineering, Construction & Assembly, Transport or Commissioning phases.

Not Applicable

Not Applicable

Not Applicable

SELLER shall include provision, to promote specific meetings with Classification Society, at the engineering and construction site.

Not Applicable

BUYER shall receive a copy of each of Classification Society monthly progress report, containing the status of all documents under approval, recommendations, outstanding items, areas of concern, surveillance status, correspondence and exchange of documents between the Classification Society and SELLER.

The monthly progress report shall also inform the certification status of the materials and equipment and other relevant items, including the Vendor’s information and status of each equipment under Classification Society approval. SELLER shall guarantee that BUYER is able to request any information directly to Classification Society regarding any equipment and/or material under SELLER´s scope of work. Confidential clauses between Classification Society and Vendors are not acceptable by BUYER.

Access to Classification Society information / documentation – SELLER shall grant to BUYER direct access to the Classification Society document electronic system.

Documentation and Design Approval for FPSO Classification

SELLER shall list all documents to be submitted to the Classification Society approval together with an issuance schedule including, drawing, plans, technical specifications, engineering procedures, calculations and studies, work execution procedures, Quality Procedures and Qualifications, Inspection Plans and other relevant to the FPSO class assignment. The list shall be approved by Classification Society And BUYER.

SELLER shall identify together with Classification Society the critical documents or “family” of documents that are critical for SELLER’s activities, so as not to impact the overall engineering and construction schedule. These documents shall be subject to a stricter and closer control regarding Classification Society review and return time.

Classification Society shall promptly inform SELLER and BUYER of changes in Rules and Regulations.

Classification Society shall advise SELLER in written form in case documents to be approved, either in full or partial, fail to meet the requirements specified in relevant Rules and Regulations. Conditions and limitations of the approval may be stated on the drawing and/or in the accompanying letter of approval. SELLER shall revise documents or perform support studies, in order to fulfill all Classification Society’s requirements.

BUYER reserves the right to monitor SELLER’s incorporation of Classification Society requirements to the documents.

Whenever plans or other documents are received for information, they shall be returned identified with approval evidence by Class Society according to current official procedures used by selected Class Society.

Approved documentation shall be identified by Classification Society stamp and signed or fully documented when in electronic format, according to procedures to be established with SELLER.

Not applicable.

SELLER shall hand over to BUYER, together with the FPSO Final Completion Term, one set of documents, in digital media (to be agreed with BUYER), including all the documents stamped by Classification Society and all letters exchanged between parties.

At FPSO handover, Classification Society shall issue the In-Service Inspection Program (IIP) as a basis for periodical and renewal class surveys during operational phase of the FPSO. SELLER, BUYER, Operation AGREEMENT and the Classification Society shall agree upon extent, frequency and inspection techniques to be described in the IIP.

SELLER shall apply to FPSO the best engineering practice and construction works, in accordance with all applicable Rules and regulations, in order to optimize In-Service Inspection Program.

Classification Society shall issue and submit to BUYER´s approval, after FPSO construction and delivery, a FPSO Descriptive Memorial Report for Brazilian Authorities containing as a minimum the following information:

- FPSO’s function, main characteristics, BUYER, design, life, dead-weight, gross and net tonnage, Register Port, class notation, characteristics of the structure, process capacity, storage tanks cubic capacity, crew, National and International applicable Rules and Regulations, energy generation, cargo equipment, mooring equipment, lifesaving equipment, fire equipment, sewage, ballast and anti-pollution systems, radio equipment, design life and list of main equipment on board.

- Hull machinery, systems and equipment maintenance condition.

- Summary report of the actual FPSO conditions.

At FPSO´s handover, SELLER shall issue the Operational Manuals of the FPSO, approved by Classification Society, BUYER and Operation AGREEMENT.

Construction supervision and surveillance for FPSO Classification

Classification Society shall carry out supervision and surveillance at SELLER’s and its Sub-SELLER’s construction sites for a period in accordance with SELLER’s schedule, in order to assure that the FPSO construction quality and procedures as well as dimensions and scantlings comply with the rules requirements and the approved documentation.

Classification Society shall carry out supervision and inspection at SELLER’s and its sub-SELLER’s construction sites, according to the approved Inspection and Tests Plan, in order to assure that specifications and fabrication procedures are being fully complied.

SELLER shall assure that, whenever required, equipment, materials, consumables and components have been certified by Classification Society, by producing original certificates for such equipment, material and components in a timely and logical manner.

Classification Society shall provide a full time surveyor, when appropriate based on the scope of work, and an adequate surveillance team at SELLER’s construction site, providing any necessary increase in team in order not to affect works performance on scheduled time.

Classification Society designated surveyor shall attend any previously scheduled meeting at SELLER’s engineering and construction site requested by SELLER’s supervision team and/or BUYER:

Extent and method of Classification Society work shall be agreed between Classification Society, SELLER and BUYER.

Provision shall be made by SELLER in order to guarantee that, if any disagreement raises between SELLER and BUYER regarding interpretation and execution of any Rules and Regulations, a Classification Society’s Main Office Senior Representative (MOSR) shall be called in to the engineering or construction site(s), at SELLER’s expense, to arbitrate and clarify such disagreement. Should no agreement between BUYER and SELLER be reached, Classification Society’s Main Office Senior Representative (MOSR) interpretation shall prevail.

SELLER shall assist the Classification Society in order to allow the proper execution of Classification Society work.

Classification Society shall comply with established working conditions and safety standards during construction, tests and trials.

Verification of Tests and Trials

Classification Society shall carry out surveillance at SELLERs and its Sub-Contractors’ construction sites in accordance with SELLER’s schedule, in order to assure that the required tests have been satisfactory carried out according to the Rules and Regulations.

SELLER shall establish the tests and trials program as required by the Rules and present, at least 24 (twenty-four) hours in advance for BUYER and Classification Society information

Confidentiality

- All documents and information provided to Classification Society shall be treated as confidential and shall not be used in any way without prior written consent of both SELLER and BUYER.

# CERTIFICATES, TERMS AND DECLARATIONS

SELLER shall provide to BUYER, before the date of towage of the FPSO to the final location, one original, one hard copy and one set in digital media (to be agreed with BUYER) of the certificates, terms and declarations as follows:

| CERTIFICATES/TERMS/DECLARATIONS | ISSUED BY |
| --- | --- |
| Services License | Coastal State Administration |
| Class certificate according to Class Notation | Classification Society |
| Tonnage Certificate | Classification Society |
| “Statement of Fact” certificate for automation, remote control and monitoring | Classification Society |
| MODU 2009 certificate | Classification Society |
| International Load Line certificate | Classification Society |
| International certificate for oil pollution prevention (Marpol Annex 1) | Classification Society |
| International certificate for sewage pollution prevention (Marpol Annex 4) | Classification Society |
| International certificate for air pollution prevention (Marpol Annex 6) | Classification Society |
| Term of compliance with ILO conventions 92 and 133 | Flag State Administration or Classification Society |
| Materials and equipment inspection certificates issued during the services for classified materials, components and equipment | Classification Society |
| Conclusion certificate | SELLER |
| Floating surveying term Administration | Coastal State |
| Certificate for helideck operation | DAerM/ANAC |
| FPSO delivery and acceptance term | SELLER |
| Guarantee term | SELLER |
| Acquaintance term | SELLER |
| Start-up trials finish declaration | SELLER |
| Final documentation delivery declaration | SELLER |
| Inventory and spare parts delivery declaration | SELLER |
| Consumable declaration (excess) | SELLER |
| FPSO manuals delivery declaration | SELLER |
| FPSO acquaintance insurance declaration | SELLER |
| Materials components and equipment guarantee transfer Term | SELLER |

SELLER shall deem that any other document, certificate or term not specified herein is relevant for the full compliance of the FPSO with the current or applicable international Rules and Regulations or best practices. SELLER shall communicate BUYER for due evaluation. The same applies to new revisions and updates the documents, certificates and terms listed.